# HC 6209 Logbook

2025.04.08 - 2025.04.14

| Proposal Title: | Temperature dependence of the non-ergodicity factor of vitreous GeO2 |
| --- | --- |
| Experimental Team: |  |
| Local Contact: | FZ |
| Filling mode and emittance: | ⅞ +1 |
| X-ray mode: |  |
| Detectors: | Eiger 4M V2 |
| Sample to Detector Distance: | 7.05 m |

# 2025.04.08

- 8:10, Beamline alignment. The machine group informed us that the beam might have moved a bit yesterday

- Tasks:

  - Check if beam moved

    - Scan #4, Psvo

    - Scan #5, psho

  - Align undulators

  - Check contrast with Vycor

  - Pilatus calibration

- Data saved in eh2_exp_0001

- Mirror initial position

  - Wmy = 0.041, wmth1 = -88.24

- Mirror final position

  - Left: initial position, right: final position; screenshots taken using only primary at 0.2x0.2

- Quick sample alignment

  - To be sure we cleanly go through the center

- Finish beam alignment

  - Tab4 in z and y done with cs at 100um

  - TS slits centered

  - Undulators (Energy of work E=9.69999KeV)

    - Written user.restore_undulators()

  - Lenses

## Mounted Lab6 to calibrate Pilatus

    - Scan 27

    - Poni file saved:

      - Lab6_eh2_exp_0001_scan0027.poni

## newsample(“vycor”)

- 12:47, mounted vycor sample to check contrast

- Data in vycor_0001

- Sample covers half of the hole. Will do scan to find edges (scan #2)

  - Scan done with 1e-3 transmission. Full beam is more than 1.07mA

  - Full beam intensity (focused): 1.68e12ph/s (@ 193mA)

- Alignment of Eige4m

  - Scan #4 and #5

# Activate delcoup

Optimize RG

EH2_EXP [72]: user_script_load("/data/visitor/hc6209/id10-coh/20250408/SCRIPTS/hc6209.py")

Dense XPCS:

EH2_EXP [73]: user.take_data_and_move(2_000,dz=0.03)

ast detectors:

scanning mode   	= TIME

acquisition time	= 0.01 sec

acquisition period  = 0.0101006 sec

acq/slow points 	= 100

number of points	= 2000  (requested 2000)

Scan 12 Tue Apr  8 15:01:08 2025 Saving in /data/visitor/hc6209/id10-coh/20250408/RAW_DATA/vycor/vycor_0001/vycor_0001.h5

EH2_EXP [82]: user.take_data_and_move(2_000,dt=0.01,dz=0.03)

Scan 13 Tue Apr  8 15:07:54 2025 Saving in /data/visitor/hc6209/id10-coh/20250408/RAW_DATA/vycor/vycor_0001/vycor_0001.h5

## XPCS saving

EH2_EXP [93]: eiger4m_v2.processing.saving_sparse.nb_frames_per_file=5000

EH2_EXP [94]: eiger4m_v2.processing.saving_dense.nb_frames_per_file=2000

# DELCOUP testing vs background

EH2_EXP [101]: delcoup.limits=(-0.5,10)

EH2_EXP [106]: delta.limits=(-0.5,10)

EH2_EXP [102]: ydet.position=0

'ydet` position reset from -803.8499999999999 to 0.0 ; offset changed from -914.9975 to -111.1475 (sign:1)

EH2_EXP [103]: ypipe.position=0

'ypipe` position reset from -681.156 to 0.0 ; offset changed from -753.72700 to -72.57100 (sign:1)

Test background (zs=-0.75)

eh2_att(1)

Delcoup = 0.5

Roi1_avg =0.50 cps/pixel

Delcoup = 1 (some diffuse rings)

Roi1_avg =2.81 cps/pixel

sct(1) scan 15

Delcoup = 1.5

Roi1_avg =0.00 cps/pixel

sct(1) scan 16

Delcoup = 1.75 (q=0.15 A-1)

Roi1_avg =0.00025 cps/pixel !!! zs = -0.75 - Vycor OUT

sct(1) scan 18

Delcoup = 1.75 (q=0.15 A-1)

Roi1_avg =15.75 cps/pixel !!! zs = 0.50 - Vycor IN

sct(1) scan 19

Delcoup = 2.33 (q=0.2 A-1)

Roi1_avg =0.0035 cps/pixel

sct(1) scan 17

At delcoup=1.75 (q=0.15 A-1) the background is in the 10-4 cps/px range !

# GeO2_6

EH2_EXP [168]: newsample("GeO2_6")

Dataset collection set to 'GeO2_6'

Data path: /data/visitor/hc6209/id10-coh/20250408/RAW_DATA/GeO2_6/GeO2_6_0001

Sample is mounted horizontally towards the machine

It seems to scatter !

Delcoup = 1.75

Sample transmission with saxs_diode

Delcoup = 0

endiode_saxs()

eh2_att(0.001)

disdiode_saxs()

We move the sample to a virgin point

EH2_EXP [253]: wm(ys,zs)

ys[mm]   	zs[mm]

--------  ----------  -----------

User

High  	14.687563   19.6876200

Current	0.100000	0.0207000

Low  	-13.312437  -16.8123800

Offset  	0.687562  -16.9967600

Dial

High  	14.000000   36.6843800

Current   -0.587562   17.0174600

Low  	-14.000000	0.1843800

### Beam size

EH2_EXP [259]: umv(delcoup,0)

EH2_EXP [260]: user.switch_to_diode()

EH2_EXP [264]: endiode_saxs()

EH2_EXP [266]: eh2_att(0.001)

Try to use the edge of the sample holder for making the edge scans

H size:

ys = -1.13

zs = -0.061 (centered)

EH2_EXP [277]: eh2_att(0.01);dscan(ys, -.1, .1, 50, 0.3); eh2_att(0.001) - Scan #18

V size:

ys = -0.361 (centered)

zs = -0.76

EH2_EXP [296]: eh2_att(0.01);dscan(zs,-.1,.1,50,0.3);eh2_att(0.001) - Scan #24

Beam FWHM: 31.8 μm x 40.8 μm (HxV)

# GeO2_6_100_C

ys[mm]   	zs[mm]

Current	0.102750	0.0083800

MACRO FOR THE NIGHT

def GeO2_6_macro():

temperatures = 170,240,310,380,450,520,590,660,730

rate = 5,5,5,3,3,3,3,3,3

i = 1

for temperature in temperatures:

print(f“set temperature = {temperature}C”)

user.set_nanodac_temp(temperature,rate[i-1],wait=True)

time.sleep(20*60)

print(f“reached temperature = {temperature}C”)

newsample(f”GeO2_6_{temperature}C”)

if i%2:

user.switch_to_transmission()

att(0.01)

dscan(ys,-0.5,0.5,50,0.2)

att(0.001)

att(0.01)

dscan(zs,-0.25,0.25,50,0.2)

goto_cen()

att(0.001)

user.switch_to_eiger()

mtimescan(0.02,60_000,1)

i = i + 1

# Setting up the Q measurements

The last measure were at

ys[mm]   	zs[mm]                                                                                                                                                                  --------  ----------  -----------                                                                                                                                                                    User                                                                                                                                                                                                High  	14.687563   19.6876200

Current	0.102750   -0.0704300

Low  	-13.312437  -16.8123800

Offset  	0.687562  -16.9967600

| delcoup | theta | Sample | bkg | Q | time |
| --- | --- | --- | --- | --- | --- |
| 1 | 2.0 | 0.120 | 0.0030 | 0.0858 | 1 h @ 1 ms |
| 1.75 | 2.0 | 0-0.7 | 0.00192 | 0.150 | 1 h @ 1 ms |
| 1.75 | 0 | 0.121 | 0.00085 | 0.150 |  |
| 3 | 1.5 | 0.160 | 0.00235 | 0.257 | 1 h @ 1 ms |
| 4 | 2.0 | 0.279 | 0.000106 | 0.343 | 1 h @ 1 ms |
| 5 | 2.4 | 0.321 | 0.000114 | 0.429 | 1 h @ 1 ms |

We take some measurement at the highlighted angles in order to obtain the dependence of the contrast on the Q

# GeO2_7 macro

205 def GeO2_7_macro():

206 	temperature  = 30, 100, 170, 240, 310, 345, 380, 415, 450, 485, 520, 555, 590, 625, 660, 695, 730

207 	rate     	=  5,   5,   5,   5,   5,   5,   5,   5,   5,   5,   5,   5,   5,   5,   5,   5,   5

208 	measure_time = 30,  30,  30,  30,  30,  60,  60,  60,  60,  60,  120, 120, 120, 150, 150, 150, 150

209

210       	# SE STOPPI CAMBIA RANGE IN RANGE(START,LEN(TEMP))

211 	for ii in range(len(temperature)):

212     	print(f"Set temperature = {temperature[ii]}C")

213     	set_nanodac_temp(temperature[ii], ramprate=rate[ii], wait=True)

214     	if temperature[ii]!=30: time.sleep(5*60)

215     	print(f"Reached temperature = {temperature[ii]}C")

216     	newsample(f"GeO2_7_{temperature[ii]}C")

217     	print(f"new measure")

218     	switch_to_transmission()

219     	eh2_att(0.01)

220     	dscan(ys, -0.5, 0.5, 50, 0.2)

221     	eh2_att(0.001)

222     	eh2_att(0.01)

223     	dscan(zs, -0.5, 0.5, 50, 0.2)

224     	eh2_att(0.001)

228     	switch_to_eiger()

229     	mtimescan(0.001, measure_time[ii]*60*1000, 1)

230     	switch_to_transmission()

231     	eh2_att(0.01)

232     	dscan(ys, -0.5, 0.5, 200, 0.2)

233     	eh2_att(0.001)

234     	eh2_att(0.01)

235     	dscan(zs, -0.5, 0.5, 200, 0.2)

236     	eh2_att(0.001)

Le misure a 450 e 485 danno un tau strano

Muoviamo il campione per cambiare punto irraggiato e prendiamo la misura a 520C

EH2_EXP [191]: umv(zs, 0)

Per sicurezza ci spostiamo ancora prima della misura a 555C

EH2_EXP [191]: umv(zs, 0.15)

EH2_EXP [192]: umv(ys, -0.52)

Il campione sembra avere un effetto di aging vicino a Tg (gobba su tau) combinato con effetti termici locali quando cambiamo spot

Facciamo alcune misure di test per capire

Mi muovo

EH2_EXP [211]: umv(ys, -0.52, zs, 0)

EH2_EXP [212]: dmesh(ys, -0.05, 0.05, 20, zs, -0.05, 0.05, 20, 0.2)

EH2_EXP [213]: mtimescan(0.001, 30*60*1000, 1)

Mi muovo

EH2_EXP [215]: umv(ys, -0.32, zs, 015)

EH2_EXP [212]: dmesh(ys, -0.05, 0.05, 20, zs, -0.05, 0.05, 20, 0.2)

EH2_EXP [213]: mtimescan(0.001, 30*60*1000, 1)

Il campione ha iniziato a cristallizzare alla temperatura di 695 C, abbiamo interrotto la misura verso la fine, cambiamo posizione per poter proseguire la rampa.

Saliamo a 730 C controllando il pilatus di tanto in tanto. Non si vedono picchi nella nuova posizione salendo in temperature.

Dopo circa 15 minuti ha iniziato a ricomparire il picco, misurando in giro per il campione lo vediamo ovunque

Decidiamo di scappare. Scendiamo a 660C

Proviamo a guardare il tau in cooling, non siamo interessati al contrasto almeno per le temperature sopra Tg.

Facciamo una misura da 60 minuti a 660C

Ad inizio misura il pilatus e` cosi`

Probabilmente si e’ cristallizzato solo il punto che abbiamo irraggiato a lungo a 695C, il picco piu’ basso che vediamo negli altri casi e’ solamente un pixel danneggiato del Pilatus.

# Cambiamo campione perche’ cristallizzato

# 11/04 GeO2_3

def GeO2_3_macro():

temperature  =  30, 100, 170, 240, 310, 380, 415, 450, 485, 520, 555, 590, 625, 660, 695, 730, 730, 660, 590, 520, 450, 380, 310, 240, 170, 100, 30, 30

rate     	=  10,  10,  10,  10,  10,  10,  10,  10,  10,  10,  10,  10,  10,  10,  10,  10,  10,  10,  10,  10,  10,  10,  10,  10,  10,  10, 10, 10

measure_time =  25,  25,  25,  25,  25,  50,  50,  50,  50, 100, 100, 100, 150, 150, 150, 180,  180, 20,  20,  20,  10,  10,  10,  10,  10,  10, 10, 20

umv(ys, 0.1, zs, 0)

for ii in range(len(temperature)):

if (ii!=16) or (ii!=len(temperature)-1):

print(f"Set temperature = {temperature[ii]}C")

set_nanodac_temp(temperature[ii], ramprate=rate[ii], wait=True)

print(f"Reached temperature = {temperature[ii]}C")

newsample(f"GeO2_3_{temperature[ii]}C")

print(f"new measure")

switch_to_transmission()

dscan(ys, -0.5, 0.5, 50, 0.2)

dscan(zs, -0.5, 0.5, 50, 0.2)

dmesh(ys, -0.05, 0.05, 20, zs, -0.05, 0.05, 20, 0.2)

eh2_att(0.001)

if ii == 16: eh2_att(0.1)

if ii == len(temperature)-1: eh2_att(0.5)

switch_to_eiger()

mtimescan(0.001, measure_time[ii]*60*1000, 1)

Macro per la notte

def GeO2_3_macro2():

temperature  =  660, 695, 730

rate     	=   10,  10,  10

measure_time =   40,  40,  30

num_rep  	=	4,   4,   5

yss      	=   .0, .05, .1,  .15,  .2

zss      	=   .1, .05, .0, -.05, -.01

#umv(ys, 0.1, zs, 0) working point

for ii in range(3):

print(f"Set temperature = {temperature[ii]}C")

set_nanodac_temp(temperature[ii], ramprate=rate[ii], wait=True)

print(f"Reached temperature = {temperature[ii]}C")

newsample(f"GeO2_3_{temperature[ii]}C")

print(f"new measure")

umv(ys, yss[ii], zs, zss[2])

switch_to_transmission()

dscan(ys, -0.5, 0.5, 100, 0.2)

dscan(zs, -0.5, 0.5, 100, 0.2)

dmesh(ys, -0.05, 0.05, 10, zs, -0.15, 0.15, 30, 0.2)

eh2_att(0.001)

switch_to_eiger()

for jj in range(num_rep[ii]):

umv(zs, zss[jj])

mtimescan(0.001, measure_time[ii]*60*1000, 1)

delcoups  	= 1,   3,  4,   5

ths       	= 2, 1.5,  2, 2.4

measure_time = 30,  30, 20,  20

print("Q measure")

newsample(f"GeO2_3_730C_Q")

umv(ys, yss[3], zs, zss[2])

switch_to_transmission()

dscan(ys, -0.5, 0.5, 100, 0.2)

dscan(zs, -0.5, 0.5, 100, 0.2)

dmesh(ys, -0.05, 0.05, 10, zs, -0.15, 0.15, 30, 0.2)

eh2_att(0.001)

switch_to_eiger()

for ii in range(len(delcoups)):

umv(zs, zss[ii])

umv(delcoup,delcoups[ii])

umv(th,ths[ii])

mtimescan(0.001, measure_time[ii]*60*1000, 1)

print("Attenuator 0.5 measure @ T = 730 C")

newsample(f"GeO2_3_730C_att2")

umv(ys, yss[4], zs, zss[2])

switch_to_transmission()

dscan(ys, -0.5, 0.5, 100, 0.2)

dscan(zs, -0.5, 0.5, 100, 0.2)

dmesh(ys, -0.05, 0.05, 10, zs, -0.15, 0.15, 30, 0.2)

eh2_att(0.001)

switch_to_eiger()

eh2_att(0.5)

for ii in range(4):

umv(zs, zss[ii])

mtimescan(0.001, 30*60*1000, 1)

Def GeO2_4_macro():

print("Set temperature = 30 C")

set_nanodac_temp(30, ramprate=30, wait=True)

time.sleep(10*60)

print("Reached temperature = 30 C")

newsample("GeO2_3_30C")

print("new measure")

umv(ys, yss[3], zs, zss[4])

switch_to_transmission()

dscan(ys, -0.5, 0.5, 100, 0.2)

dmesh(ys, -0.05, 0.05, 20, zs, -0.05, 0.05, 20, 0.2)

eh2_att(0.001)

switch_to_eiger()

mtimescan(0.001, 20*60*1000,1)

print("Attenuator 0.5 measure @ T = 30 C")

eh2_att(0.5)

mtimescan(0.001, 40*60*1000,1)

Roller blades mounted in the Ω furnace to measure beam size

Saxs_diode in

H beam size: Scan(number=8, name=dscan, path=/data/visitor/hc6209/id10-coh/20250408/RAW_DATA/GeO2_4_30C/GeO2_4_30C_0002/GeO2_4_30C_0002.h5)

V beam size: Scan(number=9, name=dscan, path=/data/visitor/hc6209/id10-coh/20250408/RAW_DATA/GeO2_4_30C/GeO2_4_30C_0002/GeO2_4_30C_0002.h5)

Beam size: 28.8 x 39.6 μm (HxV, FWHM, Pseudo-Voigt x Slit) - Same as before

Flux measurement with eh2_att(1)

sct(1) : Scan(number=10, name=ct, path=/data/visitor/hc6209/id10-coh/20250408/RAW_DATA/GeO2_4_30C/GeO2_4_30C_0002/GeO2_4_30C_0002.h5)

ch2_saxs  =    	0.00110669 A

# Focussed beam at 10.25 keV

Saxs_diode in

umv(monoe, 10.15)

U27b only

CRL3 out and check TS, CS, RG (basically aligned)

RG=CS=1

CRL3 slot#2 in

fshutopen() (needed if working only with tetramms)

EH2_EXP [521]: newsample("eh2_exp")

Dataset collection set to 'eh2_exp'

Data path: /data/visitor/hc6209/id10-coh/20250408/RAW_DATA/eh2_exp/eh2_exp_0002

V beam size vs energy

10.15 keV - 3.6 μm (FWHM, Pseudo-Voigt) - Scan 4

10.25 keV - 3.1 μm (FWHM, Pseudo-Voigt) - Scan 6

10.35 keV - 3.2 μm (FWHM, Pseudo-Voigt) - Scan 9

10.45 keV - 4.3 μm (FWHM, Pseudo-Voigt) - Scan 11

Back to 10.25 keV

10.25 keV - 3.1 μm (FWHM, Pseudo-Voigt) - Scan 13

10.25 keV - 2.5 μm (FWHM, Pseudo-Voigt with single-point derivative) - Scan 13

EH2_EXP [574]: slits

Out [574]: *** Slits ***

PS slit, (+0.100,+0.200) @ (+0.191,+0.405)

SS slit, (+1.000,+1.000) @ (+3.309,+0.075)

TS slit, (+0.200,+0.200) @ (+3.478,-0.116)

CS slit, (+0.200,+0.200) @ (+0.016,-0.001)

RG slit, (+0.400,+0.400) @ (+0.009,+0.004)

10.25 keV - 3.2 μm (FWHM, Pseudo-Voigt) - Scan 15 - V edge scan

10.25 keV - 7.6 μm (FWHM, Pseudo-Voigt) - Scan 17 - H edge scan

Beam size: 7.6 x 3.2 μm (HxV, FWHM, Pseudo-Voigt - 3 points derivative) - U27b only

Flux

sct() - Scan 18

ch2_saxs  =    	0.000341861A Undulator not all in !!!

All IDs in

Flux

sct() - Scan 21

ch2_saxs  =    	0.00122320 A

Beam size: 7.7 x 3.3 μm (HxV, FWHM, Pseudo-Voigt - 3 points derivative) - Scans 22 - 23

EH2_EXP [625]: eiger4m_v2.detector.acquisition.threshold1_energy=5100

eh2_att(0.001)

## GeO2_4_10p25

umv(ys,-0.29,zs,0.206)

C about 13% !!!

## Vycor in

Dataset collection set to 'Vycor_10p25'

Data path: /data/visitor/hc6209/id10-coh/20250408/RAW_DATA/Vycor_10p25/Vycor_10p25_0001

EH2_EXP [699]: wm(ys,zs)

ys[mm]   	zs[mm]

--------  ----------  -----------

User

High  	14.687563   19.6876200

Current   -0.140000	0.1063300

Low  	-13.312437  -16.8123800

Offset  	0.687562  -16.9967600

umv(delcoup,0)

EH2_EXP [736]: user.take_data_and_move(2_000,dt=0.01,dz=0.003)

Fast detectors:

scanning mode   	= TIME

acquisition time	= 0.01 sec

acquisition period  = 0.0101006 sec

acq/slow points 	= 100

number of points	= 2000  (requested 2000)

Slow detectors:

acquisition time	= 1 sec

acquisition period  = 1.01006 sec (requested 1.00305 sec)

number of points	= 20

Start delay   = 0.001 sec

Total time	= 20.202 sec

Scan 4 Sun Apr 13 17:40:49 2025 Saving in /data/visitor/hc6209/id10-coh/20250408/RAW_DATA/Vycor_10p25/Vycor_10p25_0001/Vycor_1

EH2_EXP [738]: eiger4m_v2.processing.saving_dense.enabled=True

EH2_EXP [739]: user.take_data_and_move(2_000,dt=0.01,dz=0.003,n_moves=6)

Fast detectors:

scanning mode   	= TIME

acquisition time	= 0.01 sec

acquisition period  = 0.0101006 sec

acq/slow points 	= 100

number of points	= 2000  (requested 2000)

Slow detectors:

acquisition time	= 1 sec

acquisition period  = 1.01006 sec (requested 1.00305 sec)

number of points	= 20

Start delay   = 0.001 sec

Total time	= 20.202 sec

Scan 5 Sun Apr 13 18:05:08 2025 Saving in /data/visitor/hc6209/id10-coh/20250408/RAW_DATA/Vycor_10p25/Vycor_10p25_0001/Vycor_10p25_0001.h5

Scan 6 with dispilatus300k()

Scan 7 with fasttimescan(2_000, 0.01) acquisition hangs !

Restart device server

Scan 8 with fasttimescan(2_000, 0.01) acquisition hangs ! Too much signal ?

Scan 9 with fasttimescan(1_000, 0.01) works !

# Focussed beam at 8.67 keV

Crl3 out

Switch_to_transmission

Det flight path has been put in air by mistake…

Go to 8.67 keV with u27b

Align TS (changes in the 10 μm range)

Realign y3 and rgvo, csvo (minor adjustments in the 10 μm range)

Roller blades on

eh2_att(1e-3)

CRL3 Slot 3 in

Align slot in ycrl3 and zcrl3

EH2_EXP [167]: newsample("eh2_exp")

Dataset collection set to 'eh2_exp'

Data path: /data/visitor/hc6209/id10-coh/20250408/RAW_DATA/eh2_exp/eh2_exp_0003

Beam size with eh2_att(0.1)

CS = RG = 1

8.67 keV - 3.4 μm (FWHM, Pseudo-Voigt, 3-point derivative) - Scan 3 - Vertical edge scan

8.67 keV - 7.9 μm (FWHM, Pseudo-Voigt, 3-point derivative) - Scan 4 - Horizontal edge scan - Bad fit !!!

Note: Scans with quite a lot of glitches

EH2_EXP [201]: slits

Out [201]: *** Slits ***

PS slit, (+0.100,+0.200) @ (+0.191,+0.405)

SS slit, (+1.000,+1.000) @ (+3.309,+0.075)

TS slit, (+0.200,+0.200) @ (+3.490,-0.147)

CS slit, (+0.200,+0.200) @ (+0.016,-0.018)

RG slit, (+0.400,+0.400) @ (+0.009,-0.015)

Put all IDs

fshutopen()

Beam size with eh2_att(0.001)

8.67 keV - 7.2 μm (FWHM, Pseudo-Voigt, 3-point derivative) - Scan 9 - Horizontal edge scan

8.67 keV - 3.4 μm (FWHM, Pseudo-Voigt, 3-point derivative) - Scan 10 - Vertical edge scan

Note: much smoother scans. Very nice derivative profiles

fshutclose()

Beam size: 7.2 x 3.4 μm (HxV, FWHM, Pseudo-Voigt - 3 points derivative) - Nice profiles !

Flux with eh2_att(1)

EH2_EXP [220]: sct()

INFO: starting Scan(number=11, name=ct, path=/data/visitor/hc6209/id10-coh/20250408/RAW_DATA/eh2_exp/eh2_exp_0003/eh2_exp_0003.h5)

ch2_saxs = 0.000964052 A

## Vycor in

EH2_EXP [226]: newsample("Vycor_8p67")

Dataset collection set to 'Vycor_8p67'

Data path: /data/visitor/hc6209/id10-coh/20250408/RAW_DATA/Vycor_8p67/Vycor_8p67_0001

EH2_EXP [280]: wm(ys,zs)

ys[mm]   	zs[mm]

--------  ----------  -----------

User

High  	14.687563   19.6876200

Current	0.000000	0.2000000

Low  	-13.312437  -16.8123800

Offset  	0.687562  -16.9967600

eh2_att(0.01)

Fast detectors:

scanning mode   	= TIME

acquisition time	= 0.01 sec

acquisition period  = 0.0101006 sec

acq/slow points 	= 100

number of points	= 2000  (requested 2000)

Slow detectors:

acquisition time	= 1 sec

acquisition period  = 1.01006 sec (requested 1.00016 sec)

number of points	= 20

Start delay   = 0.001 sec

Total time	= 20.202 sec

Scan 5 Sun Apr 13 21:12:45 2025 Saving in /data/visitor/hc6209/id10-coh/20250408/RAW_DATA/Vycor_8p67/Vycor_8p67_0001/Vycor_8p67_0001.h5

Acquisition stocks again !!!

EH2_EXP [288]: eiger4m_v2.processing.saving_dense.enabled=False

Try “sparse”  acquisition with vycor and eh2_att(1e-3)

EH2_EXP [292]: fasttimescan(2_000,0.01)

Acquisiton = 0.01 sec

Period 	= 0.0101006 sec

NPoints	= 2000

Scan mode   	= TIME

Camera mode 	= SINGLE

Start delay 	= 0.100 sec

Total Time  	= 20.301 sec

Scan 6 Sun Apr 13 21:20:07 2025 Saving in /data/visitor/hc6209/id10-coh/20250408/RAW_DATA/Vycor_8p67/Vycor_8p67_0001/Vycor_8p67_0001.h5

EH2_EXP [294]: ACTIVE_MG.disable("tetramm_diodes*")

EH2_EXP [296]: fasttimescan(2_000,0.01)

Out [296]: Scan(number=7, name=ftimescan, path=/data/visitor/hc6209/id10-coh/20250408/RAW_DATA/Vycor_8p67/Vycor_8p67_0001/Vycor_8p67_0001.h5)

EH2_EXP [297]: eiger4m_v2.detector.acquisition.threshold1_energy=4500

EH2_EXP [301]: fasttimescan(2_000,0.01)

Out [301]: Scan(number=9, name=ftimescan, path=/data/visitor/hc6209/id10-coh/20250408/RAW_DATA/Vycor_8p67/Vycor_8p67_0001/Vycor_8p67_0001.h5)

EH2_EXP [302]: user.take_data_and_move(2_000,dt=0.01,dz=0.003,n_moves=5)

Fast detectors:

scanning mode   	= TIME

acquisition time	= 0.01 sec

acquisition period  = 0.0101006 sec

acq/slow points 	= 100

number of points	= 2000  (requested 2000)

Slow detectors:

acquisition time	= 1 sec

acquisition period  = 1.01006 sec (requested 1.00016 sec)

number of points	= 20

Start delay   = 0.001 sec

Total time	= 20.202 sec

Scan 10 Sun Apr 13 21:28:04 2025 Saving in /data/visitor/hc6209/id10-coh/20250408/RAW_DATA/Vycor_8p67/Vycor_8p67_0001/Vycor_8p67_0001.h5

C about 19%

# GeO2_4_8p76

Cambiato il delcoup a 0.2 per avere lo stesso q all’interno dell’eiger

Controllato i background e sembra pulito

14/04: montata Tantala

# Beamline notes/how-to

- Chemistry lab code

  - A2601

- Accessing analysis machine

  - ssh opid10@lid10dynamix

  - Password

  - Type `exp`

  - Type `source hc6209_venv/bin/activate`

- user_script_load("/data/visitor/hc6209/id10-coh/20250408/SCRIPTS/hc6209.py")

- Programs for scan visualization

  - flint()

  - pymca()

- Transmission measurements

ACTIVE_MG.enable("tetramm_diodes*")

The diode ch2_saxs will become active for sample transmission measurements

To disable the diode during the “mtimescan”:

ACTIVE_MG.disable("tetramm_diodes*")

To see enabled (active) detectors:

ACTIVE_MG

- Changing attenuators

  - eh2_att(0.01)

- Screenshot

  - Alt+s

- Open shutter(s)

  - shopen()

- Bliss hangs

  - Open another terminal and type kill_bliss eh2_exp

  - Restart typing eh2_exp

- Move undulators back in place

  - user.restore_undulators()

- Detector problems

  - Open browser tab and go to http://tina.esrf.fr:22000/

  - Find “Eiger4M_V2”

  - Restart process (using recycle arrow)

  - Go to eh2_exp, type eiger4m_v2._det.sync_hard()

- Analysis

  - Open terminal

  - ssh -Y opid10@lid10dynamix

    - (optional)

  - Type exp

    - To change directory

  - Type id10dynamix

    - To load environment

  - Edit input file

    - May use gedit as program

  - Run analysis

    - xpcs input_filename

  - Output files are in PROCESSED_DATA

- Go home procedure

  - user.change_T(20)

  - shclose()

- Vacuum

  - Command jvacuum

EH2_EXP [484]: ACTIVE_MG

Out [484]: MeasurementGroup: measurement_eh2_exp (state='default')

- Existing states : 'default'

Enabled                               	Disabled

----------------------------------------  ----------------------------------------

eiger4m_v2:frame                      	p201_eh2_1:ct2_counters_controller:apd

eiger4m_v2:humidity                   	p201_eh2_1:ct2_counters_controller:det

eiger4m_v2:others:fill_factor         	p201_eh2_1:ct2_counters_controller:mon

eiger4m_v2:roi_counters:db_avg        	p201_eh2_1:ct2_counters_controller:sec

eiger4m_v2:roi_counters:db_max        	eiger4m_v2:input_frame

eiger4m_v2:roi_counters:db_min        	eiger4m_v2:raw_frame

eiger4m_v2:roi_counters:db_std        	eiger4m_v2:temperature

eiger4m_v2:roi_counters:db_sum        	pilatus300k:image

eiger4m_v2:roi_counters:roi1_avg      	pilatus300k:roi_counters:roi1_avg

eiger4m_v2:roi_counters:roi1_max      	pilatus300k:roi_counters:roi1_max

eiger4m_v2:roi_counters:roi1_min      	pilatus300k:roi_counters:roi1_min

eiger4m_v2:roi_counters:roi1_std      	pilatus300k:roi_counters:roi1_std

eiger4m_v2:roi_counters:roi1_sum      	pilatus300k:roi_counters:roi1_sum

eiger4m_v2:roi_counters:roi2_avg      	pilatus300k:roi_counters:roi2_avg

eiger4m_v2:roi_counters:roi2_max      	pilatus300k:roi_counters:roi2_max

eiger4m_v2:roi_counters:roi2_min      	pilatus300k:roi_counters:roi2_min

eiger4m_v2:roi_counters:roi2_std      	pilatus300k:roi_counters:roi2_std

eiger4m_v2:roi_counters:roi2_sum      	pilatus300k:roi_counters:roi2_sum

eiger4m_v2:roi_counters:roih_avg      	machinfo:current

eiger4m_v2:roi_counters:roih_max      	machinfo:lifetime

eiger4m_v2:roi_counters:roih_min      	tetramm_diag:ch1_diag

eiger4m_v2:roi_counters:roih_std      	tetramm_diag:ch2_diag

eiger4m_v2:roi_counters:roih_sum      	tetramm_diag:ch3_diag

eiger4m_v2:roi_counters:roiv_avg      	tetramm_diag:ch4_diag

eiger4m_v2:roi_counters:roiv_max      	tetramm_diag_calc:diag_intensity

eiger4m_v2:roi_counters:roiv_min      	tetramm_diag_calc:diag_y

eiger4m_v2:roi_counters:roiv_std      	tetramm_diag_calc:diag_z

eiger4m_v2:roi_counters:roiv_sum      	wcid10a:twbm_1

eiger4m_v2:roi_counters:transmission_avg  wcid10a:twbm_2

eiger4m_v2:roi_counters:transmission_max  wcid10a:twbm_3

eiger4m_v2:roi_counters:transmission_min  wcid10a:twbm_4

eiger4m_v2:roi_counters:transmission_std  wcid10a:twbm_5

eiger4m_v2:roi_counters:transmission_sum  wcid10a:twbm_6

eiger4m_v2:sparse_frame               	wcid10a:twbm_7

tetramm_diodes:ch1_sample             	wcid10a:twbm_8

tetramm_diodes:ch2_saxs               	wcid10b:t_oh1_1

tetramm_diodes:ch3_user1              	wcid10i:t_eh2_1

tetramm_diodes:ch4_user2              	wcid10i:t_eh2_2

omega_sample:omega_sample             	sens4_eh2:DiaphragmPressure

omega_body:omega_body                 	sens4_eh2:PiraniPressure

sens4_eh2:Pressure

sens4_eh2:Temperature